INSTRUCTIONS for displaying this fire shapefile in ArcMap

Open 20110912_1234-1308_Comanche_burn_ir_pl_u11n83.mxd.

This ArcMap document contains the fire shapefile color-coded according to the thermal values of the original pixels. The legend includes helpful instruction on intensity of combustion – widen the Table of Contents window to see full legend notes.

Note that the plum color is mostly ash that is heated by the sun, with limited amounts of the plum color being hot ground outside the fire area (such as unburned slopes warmed by the sun). Red colors (and oranges and yellows) are currently active fire.

Suggestion: copy/paste this color-coded fire shapefile into your ArcMap document to retain the color-coding and legend.

If the shapefile has a ‘!’ right-click on the file and go to Data, Repair Data Source (navigate to the shapefile in the directory where you unzipped the package) and click on 20110912_1234-1308_Comanche_burn_ir_pl_u11n83.shp.

Additionally, the shapefile has been ‘Saved as Layer File’ (lyr), located in the zip file, for potential use in transfer of symbology.

NOTE on selecting subsets of pixels:

If you want to select specific thermal value ranges, use item GRID_CODE.

The GRID_CODE values are listed in the legend, next to the thermal value ranges.

END.